1. IDENTIFICATION OF THE SUBSTANCE/PREPARATION AND THE COMPANY/UNDERTAKING

Änderungen:

Datum, Änderer, was?

2.4.04, MB, neu: Lagerklasse Spanien

Product information

| Trade name | STARYCIDE 480 SC |
| --- | --- |
| Product code (UVP) | 05519926, 05135869, 04982010 |
| Usage | insecticide |

| Company | Bayer CropScience AG Alfred-Nobel-Straße 50 40789 Monheim Germany |
| --- | --- |
| Telephone | +49(0)69-305-5748 |
| Telefax | +49(0)69-305-80950 |
| Responsible Department | Material and Transport Safety Management +49(0)69-305-82369/12588 |
| Emergency telephone number | +49(0)2133-51-4233 (Sicherheitszentrale Dormagen, Bayer AG) |

Telefax

### Notrufnummer - Zusatz  z.B. ORFILA

| Supplier | Bayer CropScience Deutschland GmbH Elisabeth-Selbert-Straße 4a D-40764 Langenfeld Germany Telephone: 02173 / 20760 |
| --- | --- |

2. COMPOSITION/INFORMATION ON INGREDIENTS

Chemical nature

chemische Charakertisierung

Suspension concentrate (=flowable concentrate)(SC)

Triflumuron 480 g/l

###### Gefährliche Inhaltsstoffe Neu-------------------------------------------------------------------

Hazardous components

| Chemical Name | CAS-No. / EINECS-No. | Symbol(s) | R-phrase(s) | Concentration [%] |
| --- | --- | --- | --- | --- |
| Triflumuron | 64628-44-0 264-980-3 | N | R50/53 | 39,40 wenn zusätzlich Untergrenze oder Obergrenze dann komma Untergrenze Obergrenze |
| Fatty acid methyl tauride sodium salt | 137-20-2 205-285-7 | Xi | R41 | 1,60 wenn zusätzlich Untergrenze oder Obergrenze dann komma Untergrenze Obergrenze |

3. HAZARDS IDENTIFICATION

Risk advice to man and the environment

Very toxic to aquatic organisms, may cause long-term adverse effects in the aquatic environment.

4. FIRST AID MEASURES

allgemeine Hinweise

General advice

Move out of dangerous area. If the patient is likely to become unconscious, place and transport in stable sideways position Remove soiled or soaked clothing immediately.

Einatmen

Hautkontakt

Skin contact

Wash off immediately with soap and plenty of water.

Augenkontakt

Eye contact

In case of contact with eyes rinse thoroughly with water Call a physician immediately.

Verschlucken

Ingestion

Do not induce vomiting. Wash out mouth with water. Call a physician immediately.

Hinweise für den Arzt

Notes to physician

Smptome

Treatment

Elementary aid, decontamination and symptomatic treatment.

5. FIRE-FIGHTING MEASURES

geeignete Löschmittel

Suitable extinguishing media

water spray

carbon dioxide (CO2)

foam

sand

nicht zu verwenden

besondere Gefahren bei Brandbekämpfung

Specific hazards during fire fighting

In the event of fire the following can be released:

hydrogen chloride (HCl)

hydrogen cyanide (hydrocyanic acid)

hydrogen fluoride

carbon monoxide (CO)

nitrogen oxides (NOx)

Schutzausrüstung bei Brandbekämpfung

Special protective equipment for fire-fighters

In the event of fire and/or explosion do not breathe fumes.

Use breathing apparatus.

Brandbekämpfung zusätzliche Hinweise

Further information

Contain the spread of the fire-fighting media.

Do not allow run-off from fire fighting to enter drains or water courses.

6. ACCIDENTAL RELEASE MEASURES

Personenbezogene Vorsichtsmassnahmen

Personal precautions

Avoid contact with spilled product or contaminated surfaces.

Wear personal protective equipment. Unprotected persons must be kept away.

Umweltschutzmassnahmen

Environmental precautions

Do not discharge into the drains/surface water/groundwater.

Verfahren zur Reinigung und Aufnahme

Methods for cleaning up

Take up with absorbent material (e.g. sand, earth or a proprietary absorbent material).

Clean contaminated floors and objects thoroughly, observing environmental regulations

Pack spilled material in suitable containers for recovery or disposal.

Zusätzliche Hinweise bei Freisetzung

Additional advice

Information regarding personal protective equipment, see section 8.

Information regarding waste disposal, see section 13.

7. HANDLING AND STORAGE

sieben

Handhabung

Handling

Hinweise zum sicheren Umgang

Advice on safe handling

Use only in area provided with appropriate exhaust ventilation.

Brand und Explosionsschutz

Temperaturklasse

Brandklasse

Lagerung

Storage

Anforderung an Lagerräume und Behälter

Requirements for storage areas and containers

Keep container tightly closed in a dry and well-ventilated place.

Store in a place accessible by authorized persons only.

Keep only in the original container at temperature not exceeding 50 °C.

Weitere Angaben zu Lagerbedingungen

Zusammenlagerungshinweise

Advice on common storage

Keep away from food, drink and animal feedingstuffs.

Lagerklasse gemäss VCI

| German storage class | 12 Non Combustible Liquids |
| --- | --- |

Lagerklasse Spanien

Lagerstabilität

Temperaturtoleranz

Geeignete Werkstoffe fuer Transport: Werkstoff

Suitable materials

Only use containers that are approved specifically for the substance/ product.

Ungeeignete Werkstoffe fuer Transport: Werkstoff

8. EXPOSURE CONTROLS / PERSONAL PROTECTION

acht

Ausgabe nur in der englischen Variante

Luftgrenzwerte

alt

Zusätzliche Hinweise

Technische Schutzmaßnahmen

Persönliche Schutzausrüstung

Personal protective equipment

Atemschutz

Handschutz

| Hand protection | PVC or other plastic material gloves |
| --- | --- |

Augenschutz

| Eye protection | safety glasses with side-shields |
| --- | --- |

Haut und Körperschutz

Hygienemassnahmen

Hygiene measures

Avoid contact with skin, eyes and clothing.

Keep working clothes separately.

Wash hands before breaks and at the end of workday.

Remove soiled or soaked clothing immediately and clean thoroughly before using again.

Garments that cannot be cleaned must be destroyed (burnt).

Schutzmassnahmen

9. PHYSICAL AND CHEMICAL PROPERTIES

neun

Appearance

Form

| Form | liquid, suspension |
| --- | --- |

Farbe

| Colour | white to light beige |
| --- | --- |

Geruch

| Odour | slight, original odour |
| --- | --- |

Safety  data

pH-Wert

Phasenübergang fest/flüssig

Phasenübergang flüssig/gasförmig

Phasenübergang fest/gasförmig

Flammpunkt

| Flash point | > 100 °C |
| --- | --- |

Entzündlichkeit (fest/gasförmig)

Zündtemperatur

| Ignition temperature | 485 °C |
| --- | --- |

Selbstentzündungstemperatur

Mindestzündenergie

SADT

Obere Explosionsgrenze hier

Untere Explosionsgrenze hier

Staubexplosionskennzahl

Staubexplosionsklasse (vorher Kap. 7)

Dampfdruck

Relative Dampfdichte

Dichte

| Density | ca. 1,22 g/cm³ at 20 °C |
| --- | --- |

Schüttdichte

Löslichkeit

Wasserlöslichkeit

| Water solubility | miscible |
| --- | --- |

Verteilungskoeffizient n-Octanol/Wasser

Löslichkeit in anderen Lösungsmitteln

Viskosität, dynamisch

Viskosität, kinematische

Oberflächenspannung

Schlagempfindlichkeit, nur das Ergebnis, 18.11.03

Brennzahl

Abbrandgeschwindigkeit

brandfördernde Eigenschaften

Explosivität, Methode wird ausgegeben 18.11.03

Partikelgrösse wird bis zur Klärung mit Specs entfernt 18.10.03

Staubgehalt

Sonstige Angaben

10. STABILITY AND REACTIVITY

zehn

Zu vermeidende Bedingungen

Selbsterhitzungsfähigkeit

Zu vermeidende Stoffe

Zersetzungsprodukte

Thermische Zersetzung

Gefährliche Reaktionen

- 11. TOXICOLOGICAL INFORMATION

elf

Tox akut oral

| Acute oral toxicity | LD50 (rat) > 5.000 mg/kg Test conducted with a similar formulation. |
| --- | --- |

Aktute inhalative Tox

Akute dermale Tox

| Acute dermal toxicity | LD50 (rat) > 5.000 mg/kg Test conducted with a similar formulation. |
| --- | --- |

Akute Tox. (andere)

Hautreizung

| Skin irritation | Ergebnis non-irritant (rabbit) Test conducted with a similar formulation. |
| --- | --- |

Augenreizung

| Eye irritation | non-irritant (rabbit) Test conducted with a similar formulation. |
| --- | --- |

Sensibilisierung

| Sensitization | non-sensitizing (guinea pig) OECD Test Guideline 406, Magnusson & Kligman test |
| --- | --- |

Toxizität bei wiederholter Verabreichung

Beurteilung Mutagenität

Beurteilung Kanzerogenität

Beurteilung Reprotox

Beurteilung Teratogenität

Sonstige Hinweise zur Tox

CMR-Einstufung

12. ECOLOGICAL INFORMATION

Biologisch Abbaubar

Phys.-chem Eliminerbarkeit

Bioakkumulation

Verteilung in der Umwelt

Stabilität in Wasser

Stabilität im Boden

Ecotoxicity effects

Fisch-TOX

| Toxicity to fish | Werttyp LC50 Species (Lepomis macrochirus (Bluegill sunfish)) Werte 183 mg/l Testtyp Expodauer Exposure time: 96 h Begleitanalytik |
| --- | --- |

Daphnientox

| Toxicity to daphnia | Werttyp EC50 species (Water flea (Daphnia magna)) Werte 0,00032 mg/l Testtyp |
| --- | --- |

Algentox

| Toxicity to algae | Werttyp IC50 Species (Desmodesmus subspicatus) Werte > 446 mg/l Testtyp Growth rate Exposure time: 72 h |
| --- | --- |

Bakterientox

andere terrestrische Nichtsäuger

Biologischer Sauerstoffbedarf

gelöster organischer Kohlenstoff

Chemischer Sauerstoffbedarf

Sonstige ökologische Hinweise

13. DISPOSAL CONSIDERATIONS

Dreizehn

Hinweise zur Entsorgung und Verpackung

Product

Dispose of by incineration in an authorised special waste incineration plant. Comply with local legislation.

For larger quantities contact manufacturer.

Entsorgung ungereinigter Verpackung

Abfallschlüssel

Waste key for the unused product

020108  agrochemical waste containing dangerous substances

14. TRANSPORT INFORMATION

Gefahrgutvotschriften ADR

ADR/RID/ADNR

Transport zulässig

UN-Nummer

| UN-No | 3082 |
| --- | --- |

Gefahrzettel

| Labels | 9 |
| --- | --- |

Verpackungsgruppe

| Packaging group | III |
| --- | --- |

Gefahrennummer

| Hazard no. | 90 |
| --- | --- |

UN-Bezeichner

| Description of the goods | UN 3082 ENVIRONMENTALLY HAZARDOUS SUBSTANCE, LIQUID, N.O.S. |
| --- | --- |

### Gefahrauslöser

| TestADR/RID | ( TRIFLUMURON SOLUTION) |
| --- | --- |

Ausgabe Special Provision (Sondervorschrift)

Gefahrgutvotschriften IMDG Teil 1

IMDG

Transport zulässig

| UN-No | 3082 |
| --- | --- |
| Class | 9 |
| Packaging group | III |

EmS aus Realstoff (Transportzusatzdaten)

Marine pollutant aus Realstoff (Transportzusatzdaten)

| Marine pollutant | Marine pollutant |
| --- | --- |

Gefahrgutvotschriften IMDG Teil 2

nur wenn Transport zugelassen

| Description of the goods | ENVIRONMENTALLY HAZARDOUS SUBSTANCE, LIQUID, N.O.S. |
| --- | --- |

Gefahrauslöser

|  | (TRIFLUMURON SOLUTION ) |
| --- | --- |

Gefahrgutvorschrift IATA

IATA

Transport zulässig

| UN-No | 3082 |
| --- | --- |
| Class | 9 |
| Packaging group | III |

| Description of the goods | ENVIRONMENTALLY HAZARDOUS SUBSTANCE, LIQUID, N.O.S. |
| --- | --- |

Gefahrauslöser

|  | (TRIFLUMURON SOLUTION ) |
| --- | --- |

Carriage of Dangerous Goods

Weitere Angaben für Transport (SDB)

15. REGULATORY INFORMATION

fuenfzehn

Erste Kennzeichnung

Gesetzliche Grundlage

Classification and labelling in accordance with the EC Directive on dangerous preparations 1999/45/EC as amended.

Einstufung

Classification:

Labelling according to EEC Directive

gefahrbestimmende Komponenten

Hazardous components which must be listed on the label:

- Triflumuron

Gefahrensymbole

Symbol(s)

| N | Dangerous for the environment |
| --- | --- |

Rsätze

R-phrase(s)

| R50/53 | Very toxic to aquatic organisms, may cause long-term adverse effects in the aquatic environment. |
| --- | --- |

Ssaetze

S-phrase(s)

| S35 | This material and its container must be disposed of in a safe way. |
| --- | --- |
| S57 | Use appropriate containment to avoid environmental contamination. |

allgemeine Nota

Nota zur Zubereitung

Zusatzinformation

Copra Information

Zweite Kennzeichnung

Gefahrensymbole

Dritte Kennzeichnung

Gefahrensymbole

Vierte Kennzeichnung

Gefahrensymbole

Ende der Kennzeichnungen

Kennzeichnung bestimmter Zubereitung

Besondere Kennzeichnung

Exceptional labelling of special preparations

To avoid risks to man and the environment, comply with the instructions for use.

Zusätzliche Hinweise zur Einstufung

Further information

WHO-classification: III (Slightly hazardous)

Nationale Vorschriften

National legislation

Registrierung (Zulassungsnummer) neu 20.2.04

VBF

| Risk classification according to BetrSichV (Germany) | Not subject to the regulations on combustible liquids. |
| --- | --- |

WGK

| Water contamination class (Germany) | WGK 2 water endangering |
| --- | --- |

Einstufung nach TA Luft

Vorschriften Störfall

| Legislation on the control of major-accident hazards involving dangerous substances | Subjected to the "Disasters and Major Accidents Act " Annex I, list of dangerous substances, No. 9a |
| --- | --- |

Registrierstatus

Sonstige Vorschriften (SDB)

Other regulations

BG Leaflet M053 "General Protective Measures For Working With Hazardous Substances"

Berufskrankheiten

Arbeitsmedizinische Vorsorge

16. OTHER INFORMATION

Further information

Sonstige Angaben SDB

Rsaetze

Text of R phrases mentioned in Section 2:

| R41 | Risk of serious damage to eyes. |
| --- | --- |
| R50/53 | Very toxic to aquatic organisms, may cause long-term adverse effects in the aquatic environment. |

sonstioge Angaben

Disclaimer

The advice on classification in section 15 of this Safety Data Sheet is based on European Directive on Preparations (1999/45/EC and its subsequent adaptations). The legislation on pesticide preparations should be applied by EU Member States not later than 30 July 2004.

The information given in this Data Sheet has been included in accordance with the guidelines established by  EC Directive 1991/155/EC as amended. This data sheet complements the user's instructions, but does not replace them. The information it contains is based on the knowledge available about the product concerned at the time it was compiled. Users are further reminded of the possible risks of using a product for purposes other than those for which it was intended. The required information complies with current EEC legislation. Addressees are requested to observe any additional national requirements.

This information is based on our present state of knowledge. It should not therefore be construed as guaranteeing specific properties of the products described or their suitability for a particular application.

Rsaetze

sonstioge Angaben

For further information on active ingredients see also:  Wirkstoffe in Pflanzenschutz- und Schädlingsbekämpfungsmitteln: physikalisch-chemische und toxikologische Daten IVA, Industrieverb. Agrar e.V. - 3., neubearb. Aufl. - München; Wien; Zürich; BLV Verl.-Ges.mbH, 2000 ISBN 3-405-15809-5

Disclaimer

Schluss-satz

Changes since the last version will be highlighted in the margin. This version replaces all previous versions.